# 🤖 Pi Arena Lite: Modular FMS Appliance

**A port of Cheesy Arena Lite (Team 254) for the 2026 FRC Season.**

This software turns a collection of Raspberry Pis and PCs into a professional Field Management System. It is designed to be a "Plug-and-Play" appliance for practice fields.

---

## 📂 File Directory & Purpose

### 🧠 Core Logic (The Brains)

*   **`fms_core.py`**: Runs ONLY on **Node 1 (Intel PC or Raspberry Pi)**. It manages the 160-second match clock, handles the SQLite database, and plays the official audio cues.

*   **`frc2026_node.py`**: Runs on **Nodes 2, 3, and 4**. It handles the physical world: reading sensors, pulsing LEDs, and reporting scores back to Node 1.

### ⚙️ Configuration (The Settings)

*   **`config.json`**: The "Identity Card." You edit this on each node to tell the software if it is a Red Hub, Blue Master, etc.

*   **`network_config.json`**: Tells the system how to talk to your network switch and whether you are using a PLC or Pi Nodes for scoring.

### 🌐 Web Interfaces (The Displays)

*   **`templates/audience.html`**: The big scoreboard for the crowd. Show this on Node 4.

*   **`templates/pit.html`**: The "Doctor's Office." Check this on any laptop to see if your Pis are healthy or if sensors are broken.

### 🛠️ Utilities (The Tools)

*   **`requirements.txt`**: A checklist of Python libraries that must be installed for the "appliance" to work.

*   **`deploy_field.sh` / `.ps1`**: The "Go Button." Run this once from Node 1 to wake up the entire field simultaneously.

---

## 🔌 Hardware Setup (The Appliance Wiring)

1. **Nodes 2 & 3 (Hubs)**:

- 12V Proximity Sensors → **Optocoupler** (translates 12V to Pi-safe 3.3V) → GPIO 17, 27, 22, 23.

- WS2812B LEDs → GPIO 18.

2. **Node 1 (Master)**:

- USB Panic Button.

- USB Speaker (for Match Start/End sounds).

---

## 🛡️ Reliability & "Self-Healing"

This software is designed to be an appliance.

*   **Hardware Watchdog**: If the code hangs, the Pi hardware will force a reboot within 15 seconds.

*   **Network Guard**: If a Hub loses WiFi for 60 seconds, it will automatically reboot to try and reconnect.

*   **Auto-Launch**: Using `systemd`, the software starts the moment power is applied.

---

## 📜 License

Modified BSD 3-Clause License. Portions Copyright (c) Team 254.

Designed for the **FIRST Robotics Competition 2026: REBUILT presented by Haas**.