Pi Arena Lite: Advanced Operator’s Manual (v2026.1)

A Modular Appliance Port of Cheesy Arena Lite (Team 254) for REBUILT presented by Haas.

I. PRE-FLIGHT: PLANNING & NODE ARCHITECTURE

Determine your level of play to select the appropriate Node 1 hardware and network mode.

Node Role Matrix

| Node | Location | Hardware | Primary Role |
| --- | --- | --- | --- |
| Node 1 | Home Driver Station | Intel PC (Ubuntu) | Master FMS: Timing, Audio, DB, API, Robot Control. |
| Node 2 | Red Hub (Field) | Pi 5 + Opto-Proximity | Scoring: Red Fuel counts & LED Pulse Warning. |
| Node 3 | Blue Hub (Field) | Pi 5 + Opto-Proximity | Scoring: Blue Fuel counts & LED Pulse Warning. |
| Node 4 | Visitor Driver Station | Pi 5 + USB Button | Remote Control: Start/Stop matches & Remote Audio. |
| Audience | Pit/Grandstand | Any Web Browser | Visualization: Real-time live scoreboard. |

Networking Tier Selection (config.json)

| Mode | Switch Type | Description |
| --- | --- | --- |
| FLAT | Unmanaged | Default. Single VLAN. Best for single-team practice. |
| MANUAL | Managed (Manual) | Admin assigns Ports 1–6 to VLANs 10–60 manually in the switch CLI. |
| AUTOMATED | Managed (Cisco) | Netmiko logs in and programs VLANs per-match based on schedule.csv. |

II. STAGING: HARDWARE & APPLIANCE SETUP

Wiring: 12V Proximity to 3.3V Pi

To ensure appliance longevity, use a 4-port Optocoupler for Hub nodes.

- Sensor Side: 12V V+ / GND / Signal → Opto Input.

- Pi Side: 3.3V VCC → Opto Output Common.

- Pinout (BCM):

  - Fuel Sensors: GPIO 17, 27, 22, 23 (Pins 11, 13, 15, 16).

  - WS2812B Data: GPIO 18 (Pin 12).

Appliance Reliability Config (config.json)

Enable these for "Headless" operation:

- hardware_watchdog: true (Reboots Pi if Python hangs).

- reboot_on_network_loss: true (Reboots Pi if FMS heartbeat fails for 60s).

III. THE MATCH: 160-SECOND OFFICIAL TIMELINE

The FMS automates the following sequence via piarena_fms.py.

| Match Time | Period | Red Hub | Blue Hub | Audio Cue (Node 1 & 4) |
| --- | --- | --- | --- | --- |
| 0:00 - 0:20 | AUTO | Active (2x) | Active (2x) | "Cavalry Charge" |
| 0:20 - 0:23 | TRANSITION | Buffer Only | Buffer Only | "Buzzer" |
| 0:23 - 2:10 | SHIFTS | Alternates | Alternates | "Linear Popping" (Every 25s) |
| 2:10 - 2:40 | ENDGAME | Active | Active | "Steam Whistle" |
| 2:40 | MATCH END | Inactive | Inactive | "Buzzer" |

The 3-Second "In-Flight" Rule:

- Pulse Warning: LEDs pulse during the final 3s of a shift.

- Grace Period: Hub lights turn OFF at Shift End, but sensors remain active for 3.0 seconds to count Fuel already inside the Hub.

IV. OPERATION: DATA & LOGGING

Tournament Import (schedule.csv)

Drop this file in the frc2026/ folder to automate 3v3 team tracking.

- Format: match,red_1,red_2,red_3,blue_1,blue_2,blue_3

Analytics (rebuilt_2026.db)

Matches are saved to SQLite. Use DB Browser for SQLite to view:

- red_pts vs red_balls: (Identifies teams with poor timing during inactive shifts).

- timestamp: Chronological match logging.

V. EMERGENCY PROCEDURES

The "Foghorn" Protocol (Emergency Stop)

- Trigger: Press the USB Panic Button on Node 1 or Node 4.

- Audio: The "Foghorn" cue sounds immediately.

- Robot Command: FMS sends UDP 0x04 to all Driver Stations, disabling robots instantly.

- Field State: All Hub LEDs turn RED; FMS clock freezes.

- Reset: Only the Node 1 Master can "Clear E-Stop" via the pit.html dashboard.

Manual Appliance Reset

If a Hub (Node 2/3) is misbehaving and the watchdog hasn't triggered:

- From Master PC: Run ssh pi@10.0.100.12 "sudo reboot".

- Physical: Power cycle the 12V-to-USB converter. The systemd service will resume code on boot.

VI. NETWORKING THE DRIVER STATIONS

For Pi Arena Lite to command robots (Auto/Teleop), the team laptops must be configured to "listen" to Node 1.

The DS Handshake Protocol

- IP Requirement: DS laptops must have a static IP in the 10.TE.AM.5 range (Subnet 255.0.0.0).

- FMS Recognition: The FMS (Node 1) must be at 10.0.100.5. When the DS software sees this IP, the "FMS" indicator on the dashboard turns Green.

- Firewall Rules: You must Disable Windows Firewall or create inbound rules for:

  - UDP 1121: FMS Data (The "Start" command).

  - TCP 1750: DS Status.

- Wireless Note: If using the "Flat" network mode over WiFi, teams should connect to the same SSID as the FMS. In "Automated/Manual" modes, they must plug into the physical Ethernet cables for their assigned Alliance Station.

VII. APPLIANCE NETWORK CONFIGURATION TABLES

These tables are the "Golden Record" for your field. Use these to verify that every node can talk to the FMS Master.

FMS Static IP Assignments (Subnet: 255.0.0.0 / 8)

| Device | Logic Name | Official IP | Role in Pi Arena Lite |
| --- | --- | --- | --- |
| Node 1 | piarena-fms | 10.0.100.5 | Master Brain: UDP broadcast for robots. |
| Node 2 | piarena-red-hub | 10.0.100.12 | Satellite: Reports Red Fuel scores. |
| Node 3 | piarena-blue-hub | 10.0.100.13 | Satellite: Reports Blue Fuel scores. |
| Node 4 | piarena-ds-vis | 10.0.100.14 | Remote: Visitor Control & Audio. |
| Switch | managed-switch | 10.0.100.2 | Infrastructure: VLAN destination for Netmiko. |
| PLC | field-plc | 10.0.100.50 | Legacy: Optional EtherNet/IP scoring. |

VLAN Segment Mapping (Manual/Automated Modes)

When using isolation, the switch divides the network into these digital "tubes":

| Alliance Station | Assigned VLAN | IP Subnet (Robot/DS) | Switch Port (Typical) |
| --- | --- | --- | --- |
| Red 1 | 10 | 10.R1.AM.x | Port 1 |
| Red 2 | 20 | 10.R2.AM.x | Port 2 |
| Red 3 | 30 | 10.R3.AM.x | Port 3 |
| Blue 1 | 40 | 10.B1.AM.x | Port 4 |
| Blue 2 | 50 | 10.B2.AM.x | Port 5 |
| Blue 3 | 60 | 10.B3.AM.x | Port 6 |
| Admin/Field | 100 | 10.0.100.x | All Other Ports |

VIII. PROXIMITY SENSOR CALIBRATION (APPLIANCE TUNING)

Because you are using 12V proximity sensors with an optocoupler, verify the signal logic in your piarena_node.py based on your sensor type:

- NPN (Sinking): Usually requires PUD_UP. Signal is LOW when a ball is detected. (This matches our code default).

- PNP (Sourcing): Requires an external pull-down resistor or optocoupler inversion. Signal is HIGH when a ball is detected.

- Verification: Check the Pit Dashboard (/pit). If the sensor block is Green when nothing is in the Hub, you need to invert the FALLING/RISING edge in the Python callback.

Final Project Checklist Reminder

- Node 1: PC at 10.0.100.5. Runs piarena_fms.py.

- Nodes 2-4: Pis at .12, .13, and .14. Run piarena_node.py.

- Config: config.json on Node 4 has "role": "DRIVER_STATION".

- Audio: /sounds folder copied to both Node 1 and Node 4.