Phase 1: Physical Assembly (The Hardware)

- Node 1 (Master PC): Connect USB Speakers and the USB Panic Button.

- Nodes 2 & 3 (Hubs):

  - Mount the 12V Proximity Sensors in the Hub exits.

  - Wire sensors to the Optocoupler inputs (12V side).

  - Wire Optocoupler outputs to Pi GPIO 17, 27, 22, 23 (3.3V side).

  - Connect WS2812B LEDs to GPIO 18 and 5V power from the converter.

- Networking: Ensure all nodes are on the same subnet (e.g., 10.0.100.x) via a managed switch or a dedicated router.

Phase 2: Software Provisioning (The "Appliance" Setup)

- Folder Structure: Create /home/pi/frc2026 on every Pi and copy the files from your Virtual Repo.

- Python Libraries: On every node, run:
pip install -r requirements.txt

- Enable Hardware Watchdog:

  - Add dtparam=watchdog=on to /boot/config.txt on Nodes 2 and 3.

  - Reboot the Hub Pis.

- Identity Check: Open config.json on each node and ensure the node_id and role match that specific computer.

Phase 3: Service Automation

- Create Systemd Service: Create /etc/systemd/system/frc2026.service on every node using the template provided.

- Enable Services: Run sudo systemctl enable frc2026.service on all nodes.

- Reboot All: Power cycle the entire field.

Phase 4: Pre-Match Verification

- Dashboard Check: Open http://10.0.100.5 on your laptop.

  - Are Nodes 2, 3, and 4 showing Green/Online?

- Sensor Test: Wave a ball in front of the Hub sensors.

  - Does the Pit Display show the "Beam" turning green in real-time?

- Audio Test: Use the "Test Sound" command to ensure the USB Speakers are audible.

- LED Test: Ensure Hubs are showing their correct alliance color (Red/Blue).

Phase 5: First Match Run

- Start Match: Press the USB Panic Button once.

- Autonomous (20s): Verify 2x scoring on the Audience Display.

- The Buffer: During a shift change, verify that balls scored within 3 seconds of the lights turning off still count as points.

- Match End: Verify that the match results are saved to rebuilt_2026.db.

Sleep well! Your field is ready to be REBUILT. When you wake up, do you want to start by troubleshooting the first network pings or testing the 12V proximity sensor logic?