Comparison of Cheesy Arena Lite and Pi Arena Lite:

Determining which version is "better for you" depends entirely on your specific goals: competitive event management versus practice field flexibility.

Advantages of pi Arena Lite (Modified Python Version)

The custom Python FMS is convenient for a practice environment or small-scale scrimmage because it prioritizes accessibility and specific 2026 game rules.

- Customized for REBUILT 2026: This version is hard-coded with the official 160-second match timeline, the 3-second warning pulse, and the 3-second "in-flight" buffer rule. Cheesy Arena Lite is "game-agnostic" and requires manual configuration or source code hacks to handle these specific 2026 field interactions.

- Accessibility (Python vs. Go): Python is the "lingua franca" of FRC programming. A typical student or mentor can easily modify the scoring logic, add new API endpoints, or change the LED animations. Cheesy Arena Lite is written in Go, which—while powerful—has a steeper learning curve for the average FRC team.

- Hardware "Appliance" Features: We’ve added "self-healing" features like the hardware watchdog and network-loss auto-reboot. These allow the Raspberry Pis to operate as true headless appliances that recover automatically from power or WiFi blips without human intervention.

- Open API Architecture: By building on Flask and REST/WebSockets, this version is more flexible for future expansion. A developer could build a custom mobile app or an alternative "Pit Display" just by hitting the open API endpoints we created.

- Cost-Efficient Field I/O: Original FMS and Cheesy Arena rely heavily on expensive industrial PLCs for field control. Pi Arena Lite allows you to use $80 Raspberry Pis and low-cost proximity sensors for the same high-speed scoring results.

Advantages of the Original Cheesy Arena Lite

The original Go-based software remains the industry standard for off-season competitions for specific reasons:

- Native Network Isolation: Cheesy Arena has deeply integrated logic for managing high-end Cisco switches to create secure team VLANs, preventing robots from interfering with each other—a critical feature for matches with 6 competing teams.

- Execution Performance: Go is a compiled language, meaning it is significantly faster and more efficient at handling high-concurrency tasks (like 100+ WebSocket connections) than Python's interpreted runtime.

- Deployment Simplicity: Go compiles into a single binary file. You can drop that one file onto a computer and run it without installing libraries like Flask, Pygame, or Socket.IO.

- Competition Features: Cheesy Arena includes robust match scheduling, rankings calculations, and the ability to publish results directly to The Blue Alliance.

Comparison Summary

| Feature | Pi Arena Lite | Cheesy Arena Lite |
| --- | --- | --- |
| Primary Goal | 2026 Practice & Rapid Iteration | Event Management & Robustness |
| Language | Python (Easy to read/edit) | Go (Fast/Harder to edit) |
| 2026 Logic | Built-in (Shifts & Buffers) | Manual Config Required |
| Hardware | Optimized for Pi + Proximity Sensors | Optimized for PC + PLC |
| Networking | Modular/Flexible Switches | Deeply Integrated (Cisco-centric) |
| Maintenance | Auto-healing "Appliance" mode | Manual "FMS Operator" mode |